| SPECIFICATIONS |  |  |
| --- | --- | --- |
| Interface Pin Function |  |  |
| Pin No. | Symbol | Description |
| 1 | VSS | Ground(0V) |
| 2 | VDD(3.0-5.0V) | Power supply for logic |
| 3 | V0 | Contrast Adjustment |
| 4 | RS(CS) | Data/ Instruction select signal |
| 5 | R/W(SID) | Serial data input |
| 6 | E(CLK) | Enable signal (Serial clock input) |
| 7~14 | DB0~DB7 | Data bus line33 |
| 15 | PSB | Serial/Parallel interface select |
| 16 | N.C | No connection |
| 17 | RST | reset |
| 18 | N.C | No connection |
| 19 | LED-A(3.3V) | Power supply for backlight |
| 20 | LED-K(0V) | Ground for backlight |

Outline size（mm）： 122.00 × 44.00 × 13.3 <br/>

Viewing Area：99.00 × 24.00 <br/>

Active Area： 89.55 × 18.19 <br/>

Controller IC： AIP31020 or Eqv.<br/>

Interface： SPI & Parallel <br/>

Remarks： 1×20PIN <br/>

Backlight：LED<br/>

Part Number: BGB16064-01

● Graphic LCD 160 x 32 (dots)
● FSTN-Gray / STN-Gray / STN-YG / STN-Blue
● Built-in controller (AIP31020 or Equivalent)
● +3.3V / +5.0V power supply
● 1/32 duty, 1/5bias
● Viewing Direction: 6H / 12H
● Interface: 6800 8-bit parallel
● Backlight (Side LED): White / Green / Blue / Yellow-Green / RGB / Orange / Amber
● Temp: N.T / W.T / Super W.T